## Drone Stabilization Enhancement Using PMW3901 HERE Flow Optical-FlowS ensor

## 1. Introduction

The goal of this project is to measure drift of a drone using a downward-facing camera.
Drift occurs due to wind, unstable air, or imperfect control. By tracking the movement of ground textures between video frames, we can compute how much the drone is shifting in X and Y directions.

This method is similar to how the PX4 HereFlow / PMW3901 optical flow sensors work.

# 2. Raspberry Pi Setup and SSH Connection

### 2.1 Raspberry Pi OS Boot

- Inserted Raspberry Pi3-64bit OS SD card

- Booted the Raspberry Pi

- Enabled SSH by creating an empty file named ssh in the boot partition

### 2.2 Connecting from Windows

Executed:

ssh pi@raspberrypi.local

Later switched to direct IP:

ssh pi@172.18.64.92

The Pi accepted the connection and opened the Linux terminal:

pi@raspberrypi:~ $

SSH connection successful.

# 3. System Updates

Updated the Raspberry Pi packages:

sudo apt update

sudo apt upgrade -y

This installed all latest system libraries required for camera and vision processing.

# 4. Camera Testing

Verified the camera using:

rpicam-hello

Output confirmed:

- Camera detected: ov5647 sensor

- Streaming at 30 FPS

- Exposure, gain, and ISP pipeline working

Although preview window could not open (because SSH has no display), the camera successfully captured frames.

# 5. Installing Required Libraries

Installed key packages for computer vision:

sudo apt install -y python3-opencv python3-numpy

OpenCV handles image processing and optical flow.
NumPy handles numerical calculations.

# 6. Understanding Optical Flow for Drift Measurement

Optical Flow computes how pixels move between two frames.

- If the drone moves forward → floor texture moves backward in image

- If drone drifts right → texture moves left

- By measuring this motion, we calculate drift in X and Y axes

This allows real-time drift detection without GPS or IMU.

# 7. Implementing Drift Measurement (Python Script)

Created a script (drift.py) that:

- Captures frames from the Raspberry Pi camera

- Converts frames to grayscale

- Computes Farneback Optical Flow

- Calculates movement in:

  - dx (horizontal motion)

  - dy (vertical motion)

- Computes velocity = sqrt(dx² + dy²)

# 8. Running the Drift Measurement

Executed:

python3 drift.py

The script printed lines like:

1764693548.50, 0.03, -0.01, 0.03

1764693548.72, 0.05, 0.03, 0.06

1764693549.40, 0.00, -0.00, 0.00

1764693552.58, -0.06, 0.07, 0.10

1764693558.92, 0.09, 0.01, 0.09

### Interpretation:

- dx → drift left/right

- dy → drift forward/backward

- velocity → magnitude of drift

- Near 0.00 → drone/camera stable

- Higher values → camera movement detected

These values clearly changed when the camera was moved over the ground.

# 9. Result and Verification

The results show:

- Optical flow is functioning correctly

- The camera successfully detects ground texture movement

- Drift is measurable in real time

- The sensor reacts to very small motions (<0.1 pixels/sec)

Your Raspberry Pi is now working as a custom optical flow drift sensor, similar to commercial systems used in drones.

# 10. What Has Been Successfully Achieved

✔ SSH access to Raspberry Pi
✔ Raspberry Pi OS updated
✔ Camera successfully detected and streaming
✔ OpenCV + NumPy installed
✔ Optical-flow algorithm implemented
✔ Real-time drift measurement working
✔ Numerical drift output verified
✔ System behaves like a HereFlow optical flow sensor